Identification Information

Citation

Originator: USDA Forest Service, Geospatial Technology and Applications Center, BAER Imagery Support Program

Publication Date: 2017-09-01

Title: Burned Area Reflectance Classification (BARC) Data Bundle for the DUTCHMAN Fire occurring on the Six Rivers National Forest - 2017

Publication Information

Publication Place: Salt Lake City, Utah, USA

Publisher: USDA Forest Service

Geospatial Data Presentation Form: raster digital data

Online Linkage: http://activefiremaps.fs.fed.us/baer/download.php?year=2017

Description

Abstract: These data products are preliminary burn severity assessments derived from Landsat 8 OLI data. The pre-fire and post-fire subsets included were used to create a differenced Normalized Burn Ratio (dNBR) image. The dNBR image attempts to portray the variation of burn severity within a fire. The severity ratings are influenced by the effects to the canopy.  The severity rating is based upon a composite of the severity to the understory (grass, shrub layers), midstory trees and overstory trees. Because there is often a strong correlation between canopy consumption and soil effects, this algorithm works in many cases for BAER teams whose objective is a soil burn severity assessment.  It is not, however, appropriate in all ecosystems or fires. It is expected that BAER teams will adjust the thresholds to match field observations to produce a soil burn severity.

Purpose: These data were created by the USDA Forest Service Geospatial Technology and Applications Center (GTAC) to support Burned Area Emergency Response (BAER) teams.

Supplemental Information

Fire Name: DUTCHMAN

Agency: USDA Forest Service

Land Management Unit: Six Rivers National Forest

Type of assessment: Emergency

Fire size (as mapped): 1709 acres

Pre-Fire Imagery Date and Path/Row: 2017-08-08; 045/032

Post-Fire Imagery Date and Path/Row: 2017-08-24; 045/032

Output Dataset Projection: NAD_1983_UTM_Zone_10N

Datum: D_North_American_1983

Spheroid Name: GRS_1980

Mapping Comments: The ignition location was active during pre-fire scene. The analyst felt this had minimal impact on the severity data at that location.

Product List

ca4018412336320170808_20170808_l8_refl_utm.tif = Pre-Fire Landsat 8 OLI reflectance at sensor data subset, scaled by 400 and converted to integer (band order (1-8): B, G, R, NIR, SWIR-1, SWIR-2, Coastal Blue, Cirrus). For best viewing set RGB to bands 6, 4, 3.

ca4018412336320170808_20170824_l8_refl_utm.tif = Post-Fire Landsat 8 OLI reflectance at sensor data subset, scaled by 400 and converted to integer (band order (1-8): B, G, R, NIR, SWIR-1, SWIR-2, Coastal Blue, Cirrus). For best viewing set RGB to bands 6, 4, 3.

ca4018412336320170808_20170808_20170824_dNBR.tif = Continuous dNBR product scaled by 1000; theoretical range of values is -2000 to 2000

ca4018412336320170808_20170808_20170824_dNBR_barc256_utm.tif = BARC256, 256-class (0-255); continuous image representing preliminary estimate of burn severity. This dataset can be adjusted by the user, if needed, to refine the thematic BARC4 product and/or define a new BARC4. It is anticipated that users will adjust the breakpoints between classes, then recode to their desired 3 or 4 classes.

ca4018412336320170808_20170808_20170824_dNBR_barc4_utm.tif = BARC4, Four category preliminary estimate severity classification. Subset values:

1 = unburned / undetectable (Dark Green)

2 = low severity (Cyan)

3 = moderate severity (Yellow)

4 = high severity (Red)

Time Period of Content

Multiple Dates/Times

Single Date/Time

Calendar Date: 2017-08-08 (pre-fire image)

Single Date/Time

Calendar Date: 2017-08-24 (post-fire image)

Currentness Reference: Severity data correspond to the date of the post-fire image.

Status

Progress: Evaluation of methods in process

Maintenance and Update Frequency: As needed

Spatial Domain

Bounding Coordinates

West Bounding Coordinate: 464305.670682

East Bounding Coordinate: 476335.670682

North Bounding Coordinate: 4454647.22935

South Bounding Coordinate: 4443547.22935

Keywords

Theme

Theme Keyword Thesaurus: none

Theme Keyword: Wildland Fire

Theme Keyword: Wildfire

Theme Keyword: Burned Area Emergency Response

Theme Keyword: BAER

Theme Keyword: differenced Normalized Burn Ratio

Theme Keyword: dNBR

Theme Keyword: Fire Severity

Theme Keyword: Burn Severity

Theme Keyword: USDA Forest Service

Theme Keyword: Landsat 8 OLI

Theme Keyword: Landsat 8 OLI

Place

Place Keyword Thesaurus: none

Place Keyword: Six Rivers National Forest

Place Keyword: DUTCHMAN Fire

Access Constraints: Thresholded, preliminary severity estimates (BARC4 and BARC256) are only delivered to BAER teams. Further distribution is at the discretion of the BAER team leader. Pre-imagery, post-imagery, and unthresholded data are publicly available on http://activefiremaps.fs.fed.us/baer/download.php. Access to pre-imagery and post-imagery may be limited when sensors other than Landsat or other government remote sensing assets are used due to product licensing restrictions.

Use Constraints:  There are no restrictions on use, except for reasonable and proper acknowledgement of information sources and limitations as preliminary/draft data.

Data Set Credit: USDA Forest Service

Native Data Set Environment: ERDAS Imagine, ESRI ArcGIS

Data Quality Information

Positional Accuracy

Horizontal Positional Accuracy

Horizontal Positional Accuracy Report: Landsat data are terrain corrected using a USGS digital elevation model with less than 1/2 pixel RMS error. Accuracy may vary for other sensors.

Lineage

Process Step

Process Description: These data products are derived from Landsat 8 OLI data. Pre-fire and post-fire scenes are analyzed to create a differenced Normalized Burn Ratio (dNBR) image. The dNBR image portrays the variation of burn severity within the fire. The pre- and post-fire images are terrain corrected and further processed to convert top of atmosphere reflectance.

The Normalized Burn Ratio (NBR) is computed for the pre- and post-fire images using the following formula:

(NIR Band - SWIR Band) / (NIR Band + SWIR Band) = NBR

The Differenced NBR is computed to determine severity by subtracting the post-fire NBR from the pre-fire NBR:

(PreNBR - PostNBR) = dNBR

The BARC products are a generalization of the raw, continuous dNBR dataset. Both BARC products have been resampled to unsigned 8-bit GeoTIFF and are easily viewed and edited within ArcGIS.

The classes represented on the BARC products are created with thresholds chosen by an analyst at GTAC.  These thresholds can be roughly related back to original dNBR values by multiplying by 5 and then subtracting 275 (for example, a BARC256 value of 100 would relate to a dNBR value of 225).  The BARC thresholds used on this particular fire are:

Unburned / Undetectable: All values less than or equal to 75.

Low: All values greater than 75 and less than or equal to 109.

Moderate: All values greater than 109 and less than or equal to 187.

High: All values greater than 187.

General descriptions of the severity classes are below:

Unburned / Undetectable: This means the area after the fire was indistinguishable from pre-fire conditions. This does not always indicate the area did not burn (i.e. canopy may be occluding the burn signal).

Low: This severity class represents areas of surface fire with little detected change in cover and little detected mortality of the dominant vegetation.

Moderate: This severity class is between low and high and means there is a mixture of detected effects on the dominant vegetation.

High: This severity class represents areas where the canopy has high to complete consumption.

Spatial Data Organization Information

Direct Spatial Reference Method: Raster

Raster Object Information

Vertical Count: 1

Spatial Reference Information

Horizontal Coordinate System Definition

Planar

Grid Coordinate System

Grid Coordinate System Name: NAD_1983_UTM_Zone_10N

Universal Transverse Mercator

UTM Zone Number: 10N

Transverse Mercator

Scale Factor at Central Meridian: 0.9996

Longitude of Central Meridian: -123.0

Latitude of Projection Origin: 0.0

False Easting: 500000.0

False Northing: 0.0

Planar Coordinate Information

Planar Coordinate Encoding Method: row and column

Coordinate Representation

Abscissa Resolution: 30.0

Ordinate Resolution: 30.0

Planar Distance Units: Meter

Geodetic Model

Horizontal Datum Name: D_North_American_1983

Ellipsoid Name: GRS_1980

Semi-major Axis: 6378137.0

Denominator of Flattening Ratio: 298.257222101

Distribution Information

Resource Description: Downloadable Data

Metadata Reference Information

Metadata Date: 2017-09-01

Metadata Contact

Contact Information

Contact Organization Primary

Contact Organization: USDA Forest Service, Geospatial Technology and Applications Center (GTAC)

Contact Person: Justin Epting

Contact Address

Address Type: mailing address

Address: 2222 W. 2300 S.

City: Salt Lake City

State or Province: UT

Postal Code: 84119

Contact Voice Telephone: 801 975 3755

Contact Electronic Mail Address: baerimagery@fs.fed.us

********************************************************************************

The following section should be completed after the BARC has been field validated by the BAER team and this entire metadata file should be returned to GTAC with the Soil Burn Severity data.

BAER Assessment Completion Date: November 8, 2017

BAER Team GIS Analyst: Jennifer Peterson

BAER Team Leader: John McRae

Soil Burn Severity Analyst(s): Adam Dresser

Original BARC256 thresholds:

Unburned / Undetectable: <=75

Low: >75 and <=109

Moderate: >109 and <=187

High: >187

Soil Burn Severity thresholds:

Unburned / Undetectable: <=75

Low: >75 and <=109

Moderate: >109 and <=187

High: >187

Sequence of steps used to create Soil Burn Severity data: Field verification - The Hydrologist confirmed that very few acres were burned at High Severity.  A soil scientist was not available to make the assessment.  The BARC image was converted to SBS map without changing threshold values based on input from the BAER Hydrologist who felt that it accurately reflected conditions on the ground. The Hydrologist had been assigned to the fire and had seen most of the ground prior to release of the BARC imagery.

Additional Comments: The Hydrologist had been assigned to the fire and had seen most of the ground prior to release of the BARC imagery.