R5 Klamath Happy Camp Complex 09-2014

Final Soil Burn Severity Adjustments:

I used the 0908 L8 BARC for the western watersheds of the fire.

I used the 0916 L7 BARC for the central and northeastern watersheds, and filled the data gaps with 0908 L8 data.

I used the 0916 L7 for the south and eastern watersheds, and filled the data gaps with a geoprocessing routine; the L8 data was not useful there. The result looked good on the ground.

Final breaks were as follows:

| Fire portion: | west | central+ NE | south + SE |
| --- | --- | --- | --- |
| Unb/Very Low | 0-68 | 0-57 | 0-59 |
| Low | 69-128 | 58-130 | 60-128 |
| Moderate | 129-193 | 131-225 | 129-232 |
| High | 194-255 | 226-255 | 233-255 |

I then reclassified to 4-class and combined for the final SBS map.

Notes: even the smoky areas of the L7 acquisition were pretty accurate aboveground. However there was extremely little truly high SBS due to fast fire progression and short residence time.

Adjustments made by: David Young, 530-226-2545, daveyoung@fs.fed.us